HW 3, Q1

In this study, we used the linear regression model that was retrieved by using the Python script in the book’s chapter 7.6. Our goal was to implement the trained model on microcontroller in order to estimate future room temperature values using previous temperature measurements and then comparing said temperature values with values that was predicted by the model that ran on the PC.

The original dataset was sampled at 15-minute intervals as mentioned in the book. , the data was downsampled to 1-hour intervals by selecting every fourth sample. This was done by again using the code in the book’s repository. The regression target was chosen as the Room_Temp variable. A sliding window approach was applied, where the previous five temperature values

[T(t−5),…,T(t−1)] were used as input features for predicting the next value.

The model was trained in Python and evaluated using the Mean Absolute Error metric. During the conversion of the .joblib file, an incompatibility issue was encountered when exporting the trained model using the sklearn2c library. This issue was resolved by retraining the model using sklearn2c.LinearRegressor. In the end we were able to convert the model parameters (coefficients and bias) into C source and header files.

On the microcontroller side, the regression model was implemented using simple multiply-accumulate operations. Due to floating-point type not being supported, a fixed-point arithmetic approach was adopted, where scaled integer values were used internally and formatted as decimal values for output. The microcontroller successfully generated temperature predictions, and the results were consistent with the PC-based model.

Finally, values read form the MCU were compared with the  model’s outputs that ran on the PC.