In this section, a multilayer perceptron (MLP) based Human Activity Recognition (HAR) model trained on the WISDM dataset was deployed on an STM32F746NG microcontroller using TensorFlow Lite Micro. After training in Python, the model was converted into an INT8 quantized TensorFlow Lite format and then exported as a C header file using the xxd tool. Since the microcontroller performs inference only, predefined feature vectors were used as static input arrays.

During the deployment process, several critical issues were encountered related to model conversion and project configuration. Initial attempts to export the model using SavedModel based workflows resulted in directory and compatibility errors due to TensorFlow and Keras version mismatches. These issues were resolved by directly converting the trained Keras model to TFLite format using TFLiteConverter.from_keras_model(), eliminating unnecessary intermediate steps. On the MCU side, include path problems and duplicate symbol errors were observed due to incorrect project structure and multiple main definitions. After restructuring the project and ensuring that only a single application entry point was compiled, the model was successfully deployed. As a result, the INT8 quantized model executed correctly on the STM32 platform, and inference outputs were obtained through the serial interface, confirming successful embedded deployment.